# SIH26168 — Team Explainer

AI/ML-Based Intelligent Dead Reckoning System — What We Built, Why, and How It Works

# 1. What Are We Actually Building?

This is the single most important thing for everyone on the team to be able to say in one sentence:

The app tells you where you are, continuously, even after GPS is lost. It does NOT tell you which way to turn to reach a destination.

These are two different problems:

- Positioning (what we built): "You are here, and here's how confident we are in that estimate." This is the map, the dot, the growing uncertainty circle.

- Routing / turn-by-turn (NOT what we built): "Turn left in 200m." Google Maps already does this well — it just breaks the moment GPS disappears, because its position estimate goes bad.

Our app solves the upstream problem that routing apps depend on: keeping a reliable position estimate alive through a GNSS blackout (tunnels, underground parking, dense urban canyons), so that whatever app is doing the routing doesn't lose the thread.

If a judge asks "does your app give directions?" — the honest, correct answer is no. It keeps you located; routing is a separate, already-solved problem we deliberately did not rebuild.

# 2. Why Is This Problem Genuinely Hard?

This is an active academic research area — the original problem statement cited papers from 2021–2025 still working on exactly this. That's why ISRO posed it as a hackathon challenge rather than expecting a finished commercial product. The difficulty is real; the fix was to keep the SYSTEM simple even though the underlying MATH (sensor fusion under uncertainty) is hard.

Every one of the following was a deliberate scope decision, made to keep the project buildable in the time available:

- Single smartphone only — no dedicated hardware, no vehicle CAN-bus access.

- One trained model, one deployment domain — vehicle-only (matches the only real, audited accuracy figure we have).

- No backend, no multi-user system, no admin dashboard.

- No pedestrian/walking mode (descoped after the trained pedestrian model was found to be unused and unwired).

- One city's offline map (Coimbatore) instead of statewide coverage (a full Tamil Nadu map was investigated and found to cost 3.5GB+ with real engineering risk, for no real demo benefit).

None of these are things we "couldn't" do — they're things we chose not to do, so the parts we DID build would actually work and be defensible under questioning.

# 3. Why No Backend?

Because the entire value of the system depends on working with ZERO connectivity — that is the whole point of a GNSS-denied environment. If position tracking required phoning a server, it would break exactly when it's needed most (inside a tunnel with no signal at all).

So all sensor fusion, ML inference, and map matching run entirely on-device, by design — not as a shortcut, but because a backend would be actively wrong for this specific problem.

# 4. Why No Admin Dashboard / React Web App?

An admin dashboard implies a backend that aggregates multiple users' positions across a fleet — that's a genuinely different product (a logistics/fleet-management platform), not what this problem statement asks for.

It's not a bad idea — fleet tracking is listed as a real future deployment path (ride-hailing, delivery, freight) — but building it now would mean building an entire second system, with its own backend, under deadline pressure, for a demo that doesn't need it. This is a strong bullet point for the "Future Work / Impact" slide, not something to build before the deadline.

# 5. System Architecture — How It Actually Works

## Tech Stack

- Platform: Native Android (Kotlin, Jetpack Compose) — fully on-device.

- Sensor Fusion: Extended Kalman Filter (EKF, 6-state) + Zero-Velocity Update (ZUPT) + Non-Holonomic Constraint (NHC, vehicle motion) + magnetometer-fused heading with live confidence monitoring.

- Machine Learning: 2-layer GRU neural network, deployed on-device via ONNX Runtime. Trained on IO-VNBD — the real vehicle inertial-odometry dataset officially linked on the SIH portal for this exact problem statement.

- Mapping: Offline OpenStreetMap tiles + road network, with map-matching to snap raw position onto the most plausible real road — works with zero connectivity.

- Data: IO-VNBD (primary) + OxIOD (supplementary) — both real, published, peer-reviewed inertial-odometry research datasets.

## System Flow

- 1. Sensors (accelerometer, gyroscope, magnetometer, GPS) feed continuously into the sensor fusion layer.

- 2. The EKF (with ZUPT and NHC) maintains a running position + velocity estimate.

- 3. While GPS is available, it directly corrects the estimate.

- 4. When GPS is lost (blackout), the on-device GRU model predicts short-term displacement from recent motion data.

- 5. A kinematic gate checks every ML prediction against a physically plausible envelope before accepting it — this stops a bad AI guess from corrupting the position.

- 6. The fused estimate, plus a growing confidence radius, is map-matched onto the nearest real road and shown live.

- 7. When GPS returns, the estimate is smoothly reconciled with the fresh fix rather than jumping discontinuously.

# 6. UI Walkthrough — What Every Number Means

The current screen shows a lot at once. Here is what each part actually means, in plain language:

| UI Element | What It Means |
| --- | --- |
| GNSS / DEAD RECKONING badge | Which mode is currently active — using real GPS, or running on phone sensors alone because GPS is gone. |
| GNSS BLACKOUT + Duration | How long the app has been running without GPS. |
| DR Distance | How far the app thinks you've traveled since GPS was lost, based on sensors alone. |
| Max Error | An evaluation-only number (compares against real GPS still running silently in the background for testing). Would not exist in a real tunnel — it's how WE check accuracy, not something a real user sees. |
| ML Latency | How fast the AI model runs, in milliseconds. Proves it is genuinely running on-device, not simulated. |
| Confidence Radius | "You are probably within this many meters." This is the honest, real-world-usable uncertainty measure — unlike Max Error, it works with no secret GPS access. |
| Heading Conf. | How much the app currently trusts its own sense of direction (LOW / MEDIUM / HIGH). A wrong sense of direction throws off everything downstream. |
| GNSS (status row) | Is real GPS currently available. |
| Navigation (status row) | Which system is actively driving the position estimate right now — GNSS or DR. |
| Motion | Is the phone detected as stationary, moving, or rotating. |
| ML Gate (Accepted/Clamped/Rejected) | How many of the AI model's suggested corrections were accepted vs. rejected as physically implausible. This is the safety check protecting the position estimate from bad AI guesses. |
| ML / EKF (status) | Confirms the AI model and the core math engine are both actively running. |
| MAP / OFFLINE MAP | Confirms the map works with zero internet connectivity. |
| Position (lat/long) | Raw GPS-style coordinates of the current estimate. |
| Speed / Heading / Distance | Current motion readout. |
| DR Error / Drift | Accuracy statistics for the current session. |
| Sensor Status | Confirms the phone's accelerometer, gyroscope, and magnetometer are actively supplying data. |

A simplified two-view redesign is in progress: a plain "Navigating without GPS" screen with just a map and a High/Medium/Low confidence label for everyday users, with all of the above moved behind a "Show technical details" toggle for judges/engineers who want the full picture.

# 7. Known Accuracy & Honest Limitations

This is the most important section to get right if asked directly — being honest about limitations is a strength in this project's story, not a weakness, and matches how every audit along the way has been handled.

- Real, audited result: 11.28m mean error, a 54.45% reduction vs. an uncorrected baseline, measured on 27,964 genuinely held-out test windows from IO-VNBD.

- The critical caveat: this figure was measured WITH accurate heading (from the vehicle's own CAN-bus/dual-antenna GPS during testing). Our actual phone-only app has no CAN-bus — it relies on the phone's own magnetometer-fused heading, whose reliability specifically inside a vehicle or a tunnel is not fully characterized.

- Correct framing to use: present 11.28m as "ML displacement-model accuracy, evaluated with reference heading," and separately, explicitly disclose that end-to-end on-device accuracy depends on phone heading quality, which has not yet been isolated or measured. This is not a new admission — it's a limitation the source research itself already discloses.

- Current deployment scope: vehicle-only. Pedestrian/walking use is explicitly future work, not implemented.

# 8. Frequently Asked Questions

## "Isn't this already solved by Google Maps?"

Not for this configuration. Mature automotive dead-reckoning solutions exist, but they depend on dedicated hardware and vehicle CAN-bus access. This problem specifically asks for a bare-smartphone solution, which remains an active, unsolved research area.

## "Why not just use GPS or NavIC directly?"

No satellite system can penetrate tunnels, underground structures, or dense high-rises — that's a physical limitation, not an engineering gap satellites can close.

## "How accurate is it, really?"

See Section 7 above — report the number with the heading caveat, every time, not just once in fine print.

## "Why did we cut [backend / pedestrian mode / statewide map]?"

Each was a deliberate scope decision to keep the shipped system reliable and defensible under a hard deadline, not a sign of giving up on the idea — each is a legitimate, documented "future work" item.

## "What happens when GPS comes back?"

The system reconciles the dead-reckoned estimate with the fresh GPS fix smoothly, rather than snapping the position discontinuously.

Prepared as a team-shared explainer — read before pitch rehearsal so everyone can answer any of the above in their own words.